Advanced Forest Fire Prediction System

From School Project to Systems Engineering

| Student [Your CAO Number] | Word Count ~790 words | Submission ISE Route A — 200pts |
| --- | --- | --- |

Section 1

Overview of Work

This project began as a standard Leaving Certificate coursework assignment—a basic Micro:bit temperature logger—but I have completely re-engineered it into an advanced environmental monitoring system. My goal was to move beyond simple data logging and build a predictive engine capable of calculating real-world fire risk using industry-standard models.

The system consists of two integrated components:

- Hardware Edge Device:  A BBC Micro:bit programmed in MakeCode to collect real-time temperature, light, and soil moisture data. It transmits this telemetry over a serial connection (115200 baud) to the desktop.

- Software Processing Unit:  A custom Python application (built with Tkinter, Pandas, and Matplotlib) that processes the live data stream, visualises the data, and calculates the fire danger rating.

Technical Deep Dive: The McArthur Index

The core of the system's intelligence is the implementation of the McArthur Forest Fire Danger Index (FFDI)—the official mathematical model used by Australian fire services. Unlike my original project, which used a simplistic linear formula (temp * 0.6), this version implements the actual differential equation:

# McArthur Forest Fire Danger Index (FFDI) implementation

# Clamping to max(1) prevents math domain errors in log()

drought_factor = max(1,  10 - (soil_moisture / 10))

ffdi = 2 * math.exp(

-0.45  +  0.987 * math.log(drought_factor)

- 0.0345 * humidity  +  0.0338 * temp  +  0.0234 * wind_speed

)

Implementing this was my most significant technical challenge. Initially, the application would crash with a ValueError: math domain error. By debugging, I realised that when soil moisture was high, the calculated drought_factor could drop to zero or negative—since the logarithm of a non-positive number is undefined, this crashed the engine. I solved this with clamping logic (max(1, ...)), ensuring mathematical inputs remain valid under all sensor conditions—a lesson in defensive programming for critical systems.

Machine Learning Integration

My second major addition was a Machine Learning Predictor. I researched time-series forecasting techniques to understand the concept, then implemented the solution manually using scikit-learn. My first attempt produced a useless flat line because it treated days as independent numbers. To fix this, I engineered a 7-Day Sliding Window approach: by training the model on the previous 7 days to predict the 8th, the system learns to recognise trends rather than just absolute values—enabling accurate 7-day fire risk forecasting.

📷 INSERT — Screenshot of MakeCode blocks & Micro:bit hardware setup

Section 2

Project Impact

My motivation for this project stems from the devastating 2019–20 Australian bushfire season. Watching the scale of that destruction made me realise that climate disasters are not abstract—they are data problems that can be modelled. I wanted to build a tool that didn't just react to fire, but predicted it.

Climate Change Simulation

The most impactful feature is the Climate Change Simulation. I built a set of controls that allow users to apply environmental offsets (e.g. +2°C temperature). When I ran this on my synthetic dataset, the results were stark: a mere 2°C rise didn't slightly increase risk—it quadrupled the number of days classified as "Severe" or "Catastrophic". This transformed the project from a coding exercise into a functional climate analysis tool.

Adaptive Alertness Algorithm

To make the system practical in a real monitoring context, I developed an Adaptive Alertness Algorithm. Raw sensor data is noisy—alarms can flicker annoyingly. I designed an asymmetric smoothing approach:

- Slow increase (0.1 factor) — prevents false positives from momentary spikes.

- Fast decrease (0.5 factor) — ensures the "All Clear" is given quickly once danger passes.

This attention to the human side of the data—preventing alarm fatigue—is what makes the system practical.

📷 INSERT — Screenshot of ML Prediction Window (7-Day forecast graph)

📷 INSERT — Screenshot of Main Dashboard with climate simulation sliders & danger zones

Section 3

Reflection and Improvements

User-Centric Iteration

User testing was critical to the project's evolution. Feedback from a peer highlighted that the standard Tkinter design looked outdated and the raw risk numbers were hard to interpret. In response, I upgraded the GUI using ttkthemes ("breeze" theme) for a modern look and added colour-coded Danger Zones (Green → Red) to all graphs, making the data instantly readable for non-experts.

Data Integrity Lessons

One of the hardest hidden bugs involved data integrity. The Python serial reader kept crashing with UnicodeDecodeError. I discovered that the Micro:bit was sending the ° symbol as extended ASCII, conflicting with Python's UTF-8 decoder. I wrote a byte-level filtering routine to strip non-standard characters before processing. This taught me that in real-world engineering, you cannot trust external data sources to be clean—you must sanitise everything.

Future Vision

If I were to extend this project, I would integrate LoRa radio modules to allow multiple Micro:bits to transmit data over kilometres to a central hub, and visualise this sensor network on a Folium map dashboard. This would evolve the system from a single-point sensor into a distributed area-monitoring network.

Overall, this project transitioned me from simply writing code to engineering a robust, integrated system that addresses a real-world problem—and left me hungry to take it further.

🔗  Full Source Code: [INSERT GitHub Repository URL — ensure public access is enabled]